Week 1 – Layered Network Architecture

| Layer | Primary Role | Key Concepts |
| --- | --- | --- |
| Transport Layer | Process-to-process communication | Uses port numbers to deliver data to the correct application. Examples include TCP and UDP. Handles reliability, flow control, and multiplexing. |
| Network Layer | Host-to-host communication | Uses logical addressing (IP) to deliver packets across networks. Responsible for routing decisions and packet forwarding. |
| Data Link Layer | Node-to-node communication | Uses MAC addresses for delivery on the same physical link. Handles framing, error detection, and medium access control. |

Week 2 – Ethernet & Data Link Layer Concepts

| Topic | Key Concepts |
| --- | --- |
| Ethernet Frame Size | Minimum payload of 46 bytes ensures CSMA/CD collision detection in early Ethernet. Maximum payload of 1500 bytes chosen for historical cost and fairness reasons. Jumbo frames exist today but standard MTU remains 1500 bytes. |
| Ethernet II vs IEEE 802.3 | Values ≤1500 interpreted as length (IEEE 802.3). Values ≥1536 interpreted as type (Ethernet II). IP traffic uses Ethernet II (e.g. 0x0800). |
| Ethernet Topology | Modern Ethernet uses star or extended star topology. Hubs create shared collision domains; switches isolate collision domains per port. |
| MAC Protocol (CSMA/CD) | Required only in half-duplex shared media. Disabled in full-duplex switched Ethernet. Collisions are not expected in modern networks. |
| Ethernet Multicast Address | Multicast MAC addresses have the least significant bit of the first octet set to 1. Used for one-to-many Layer 2 delivery. |
| Collision Domain | Area where frames can collide. Reduced by switches; eliminated per switch port. |
| Broadcast Domain | Area where broadcast frames are received by all nodes. Defined by VLANs and broken by routers or Layer-3 boundaries. |

Week 3 – Network Layer & IPv4

| Topic | Key Concepts |
| --- | --- |
| Core Functions of IP | Addressing identifies hosts, routing determines next hop, fragmentation allows packets to traverse links with smaller MTUs. Reassembly occurs only at the destination. |
| IP Characteristics | Connectionless, best-effort, no guarantees of delivery, order, or duplication. Reliability handled by higher layers such as TCP. |
| IPv4 Header – Version & Length | Version field identifies IPv4. Header length specifies header size in multiples of 4 bytes (minimum 20 bytes). |
| IPv4 Header – TTL | Prevents infinite routing loops by decrementing at each hop. Packet is discarded when TTL reaches zero. |
| IPv4 Header – Protocol | Identifies upper-layer protocol (ICMP = 0x01, TCP = 0x06, UDP = 0x11). |
| IPv4 Packet Size | Total length field allows up to 65,536 bytes, but practical size limited by MTU. |
| IPv4 Address Types | Includes public, private (RFC 1918), loopback, link-local, multicast, and broadcast addresses. |

Week 4 – Network Design, VLANs & Inter-VLAN Routing

| Topic | Key Concepts |
| --- | --- |
| Hierarchical Network Model | Access layer connects end devices. Distribution layer handles routing and policy. Core layer provides high-speed backbone connectivity. |
| Collapsed Core Design | Combines core and distribution layers. Common in small to medium networks to reduce cost and complexity. |
| VLAN Concept | VLANs logically segment a network at Layer 2, reducing broadcast domains without changing physical topology. |
| VLAN Tagging (802.1Q) | Frames are tagged when traversing trunk links. Sending switch inserts tag; receiving switch removes it. |
| Native VLAN | Allows untagged frames on trunks for backward compatibility. Native VLAN mismatch can cause security and traffic leakage issues. |
| Access vs Trunk Ports | Access ports carry one VLAN. Trunk ports carry multiple VLANs between switches. |
| Dynamic Trunking Protocol (DTP) | Dynamic auto waits passively. Dynamic desirable actively negotiates trunking. Best practice is to disable DTP where possible. |
| Legacy Inter-VLAN Routing | One physical router interface per VLAN. Poor scalability and rarely used today. |
| Router-on-a-Stick | Single trunk link with router subinterfaces. Cost-effective but limited by router throughput. |
| Layer-3 Switch Inter-VLAN Routing | Uses SVIs for each VLAN. Fastest and most scalable solution in modern networks. |

Week 5 – Address Resolution Protocol (ARP)

| Topic | Key Concepts |
| --- | --- |
| Purpose of ARP | Resolves an IP address to a MAC address so Layer-3 packets can be delivered on Ethernet. |
| ARP Request | Broadcast message sent to all devices in the broadcast domain asking for the MAC address of a target IP. |
| ARP Reply | Unicast response sent by the device that owns the target IP, providing its MAC address. |
| ARP Packet Structure | Includes hardware type, protocol type, hardware length, protocol length, operation code, sender and target addresses. Same format for request and reply. |
| ARP Cache | Temporarily stores IP-to-MAC mappings to reduce broadcast traffic. Entries age out over time. |
| Hop-to-Hop vs End-to-End | MAC addresses change at every hop. IP addresses remain constant from source to destination. Switching is hop-to-hop; routing is end-to-end. |

| Type of EtherChannel configs |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| EtherChannel Method |  | Negotiation? | Standard/Proprietary |  | Key Notes |  |
| Static (Manual) |  | No | Vendor-specific configuration |  | Interfaces are manually forced into a channel-group; no protocol checks/negotiation. |  |
| PAgP |  | Yes | Cisco proprietary |  | Uses PAgP to negotiate channel formation; both ends must support PAgP. |  |
| LACP |  | Yes | IEEE 802.3ad |  | Standards-based negotiation; preferred for multi-vendor interoperability. |  |
| EtherChannel Type | Typical Topology Use |  |  | Interface Mode |  | What It Carries / How It’s Used |
| Layer 2 EtherChannel | Access ↔ Distribution |  |  | Trunk (switchport) |  | VLAN-tagged traffic across multiple VLANs (supports inter-VLAN designs). |
| Layer 3 EtherChannel | Distribution ↔ Core |  |  | Routed port (no switchport) |  | Point-to-point routed connectivity; interfaces behave like router interfaces and can be assigned IP addresses. |

Rapid Spanning tree protocol (RSTP)

| Election Process |  |  |  |
| --- | --- | --- | --- |
| What is Being Elected | Primary Decision Metric | Tie-Breakers (in order) | Resulting Role/Outcome |
| Root Bridge (Root Switch) | Lowest Bridge ID (BID) (priority + VLAN ID + MAC) | Lowest priority → lowest MAC address | One switch becomes Root and originates the “best” BPDUs (root info). |
| Root Port (per non-root switch) | Lowest total path cost to root: (received BPDU root path cost + local interface cost) | 1) Lowest sender BID (of received BPDU) → 2) Lowest sender port priority (Port ID) → 3) Lowest sender port number (Port ID) | Each non-root switch selects one Root Port (best upstream path). |
| Designated Port (per segment/link) and Alternate Port | Port that advertises lower root path cost (better BPDU) | If cost ties: lower BID wins Designated | Designated forwards for that segment; the other side becomes Alternate and goes Discarding/Blocking (loop prevention, standby). |

| Port Roles |  |  |  |
| --- | --- | --- | --- |
| Port Role | Where It Exists | What It Does | Typical State |
| Root Port (RP) | Non-root switches only | Best path toward the Root Bridge (one per non-root switch, per VLAN) | Forwarding |
| Designated Port (DP) | Root + non-root switches | Forwards for a segment; best port away from root on that LAN segment | Forwarding |
| Alternate Port (AP) | Non-root switches only | Backup path toward root; prevents loops by standing by | Discarding (often shown as BLK) |
| Backup Port (BP) (rare) | Non-root switches (uncommon) | Backup to a designated port on the same switch for the same segment | Discarding |

| Port States |  |
| --- | --- |
| STP (802.1D) States | Closest RSTP (802.1w) Equivalent |
| Blocking + Listening | Discarding |
| Learning | Learning |
| Forwarding | Forwarding |
| Disabled | (Not a defined RSTP state; effectively non-participating/administratively down) |

| FHRP generic properties |  |  |
| --- | --- | --- |
| Property | What It Means | Key Notes / Typical Values |
| Full Name | First-Hop Redundancy Protocol (FHRP) | A category of protocols (not a single protocol) |
| Primary Purpose | Default gateway redundancy | Prevents loss of upstream connectivity if the gateway device/link fails |
| Network Layer Focus | Layer 3 gateway function (with Layer 2 impact) | Hosts route to a gateway IP; switches forward to a gateway MAC |
| Core Mechanism | Virtual gateway identity | Presents a Virtual IP (VIP) (and usually Virtual MAC (VMAC)) shared by multiple devices |
| Host Configuration Model | Hosts point to the VIP as default gateway | Hosts do not reconfigure during failover |
| Device Roles | Active/Standby (or Master/Backup) | One forwards traffic; others monitor and are ready to take over (some FHRPs can do load-balancing) |
| Failover Trigger | Loss of protocol hellos / timers (or tracked object failure) | If the active stops advertising, a standby takes over; many designs also track uplinks |
| Failover Behavior | Standby assumes VIP/VMAC ownership | Often uses gratuitous ARP (or equivalent) so switches relearn the VMAC location quickly |
| Traffic Forwarding | Usually only the active forwards for that VIP | Depends on protocol: some support per-host/per-flow distribution (e.g., GLBP) |
| Common Deployment Point | Distribution layer SVI/gateway per VLAN | Frequently used as the default gateway for VLANs in campus designs |
| Load Sharing Option | Possible, depending on protocol/design | Common method: make different devices active for different VLANs (HSRP/VRRP); or protocol-native (GLBP) |
| Typical Examples | HSRP, VRRP, GLBP | HSRP/GLBP are Cisco; VRRP is standards-based |
| Convergence Goal | Minimal downtime and transparent recovery | Keeps gateway identity stable so users “don’t notice” a gateway failure |

| HSRP properties |  |  |
| --- | --- | --- |
| Property | What It Is | Key Values / Notes |
| Protocol Type | First-Hop Redundancy Protocol (FHRP) | Provides default-gateway redundancy at the first hop |
| Vendor / Standard | Cisco-proprietary | HSRP is Cisco-specific (commonly deployed on Cisco networks) |
| Core Concept | Virtual default gateway | Hosts use a Virtual IP (VIP) as their default gateway |
| Virtual IP (VIP) | Shared gateway IP address | Stays the same during failover (no host reconfiguration) |
| Virtual MAC (VMAC) | MAC address mapped to the VIP | Auto-generated by HSRP; used by hosts/switches at Layer 2 |
| Router Roles | Active / Standby | Active forwards gateway traffic; Standby monitors and takes over on failure |
| Failure Detection | Periodic HSRP packets (hellos) | If the active stops sending, standby assumes failure and becomes active |
| Failover Method | Gratuitous ARP (GARP) | New active sends unsolicited ARP reply to claim VIP/VMAC ownership |
| Switch Table Impact | CAM/MAC table update | Switches relearn VMAC → new port, redirecting traffic instantly |
| Host Impact | Minimal / transparent | Default gateway remains VIP; ARP mapping typically remains consistent |
| Common Deployment | Distribution layer default gateway for VLANs | Often paired with redundant distribution switches |
| Load Balancing Approach | Active role split across VLANs | Make one switch active for some VLANs and the other active for others to share load |

Week 7

| OSPF overview |  |  |
| --- | --- | --- |
| Item | Key Point | Notes |
| Protocol type | Link-state IGP | Routers learn topology, then compute best paths using Dijkstra (SPF) |
| Version (IPv4) | OSPFv2 | OSPFv3 is for IPv6 (not covered here) |
| IP transport | Runs directly over IPv4 | IP Protocol number = 89 |
| Multicast destinations | 224.0.0.5 (All OSPF routers), 224.0.0.6 (All DR/BDR) | Used for Hello and other OSPF control traffic |
| Administrative Distance | 110 | Shown in routing table as [110/metric] |
| Core phases | 1) Neighbor discovery 2) Topology exchange 3) SPF calculation | Matches your notes structure |

| Packet Types |  |  |
| --- | --- | --- |
| OSPF Type | Name | Purpose (What It Does) |
| 1 | Hello | Discover/maintain neighbors; DR/BDR election on multiaccess networks |
| 2 | DBD (Database Description) | Summarizes LSDB contents using LSA headers only (efficient comparison) |
| 3 | LSR (Link State Request) | Requests missing/newer LSAs from a neighbor |
| 4 | LSU (Link State Update) | Sends LSAs (full details) requested or flooded |
| 5 | LSAck (Link State Acknowledge) | Acknowledges receipt of LSUs/LSAs |

## Phase 1: Discover Adjacent Neighboring Routers

- Enable OSPF on interfaces (via network … wildcard … area … or ip ospf … area …).

- Interface state transitions Down → Init and starts sending Type 1 Hello packets.

- Hello neighbor discovery logic

  - If no Hellos received yet: sends Hello with no neighbor list.

  - After receiving Hellos: includes neighbor Router IDs in the Hello Neighbor field.

  - When a router receives a Hello that lists its own Router ID, it reaches 2-Way state.

- Multiaccess Ethernet optimization (DR/BDR)

  - To reduce excessive exchanges, routers elect a Designated Router (DR) and Backup Designated Router (BDR).

  - Non-DR routers (DROTHER) primarily exchange OSPF info with DR/BDR.

  - Election criteria: highest OSPF priority wins (default 1), then highest Router ID if tied.

  - Election is non-preemptive after completion (new arrivals don’t take over).

## Phase 2: Learn Network Topology

- Routers move from 2-Way → ExStart → Exchange to begin database synchronization.

- Exchange Type 2 DBD (Database Description) packets:

  - DBD contains LSA headers (summaries) so routers can compare databases efficiently.

- If differences exist, routers enter Loading:

  - Type 3 LSR requests missing/newer LSAs.

  - Type 4 LSU sends the requested LSAs (updates).

  - Type 5 LSAck acknowledges LSUs.

- When link-state databases are synchronized, neighbors reach Full state.

## Phase 3: Compute Lowest-Cost Paths to Remote Subnets

- Each router independently runs Dijkstra (SPF) on the learned topology (LSDB).

- Computes lowest-cost paths based on summed interface costs.

- Installs best routes into the routing table:

  - OSPF-learned routes marked O.

  - Metric shown as [110/cost] where 110 is OSPF administrative distance and cost is total path cost.

- If multiple equal-cost best paths exist, OSPF can install multiple routes (ECMP) for load balancing.

| Topic (from doc) | Configuration Commands | Verify / Debug / Clear Commands |
| --- | --- | --- |
| Access vs Trunk Ports + DTP | Access port: int gi0/1switchport mode accessswitchport access vlan 10 Trunk port: int gi0/24switchport mode trunkswitchport trunk allowed vlan 10,20switchport trunk native vlan 99 Disable DTP: int gi0/24switchport nonegotiate | show vlan briefshow interfaces trunkshow interfaces gi0/24 switchport |
| Inter-VLAN Routing (Router-on-a-Stick / L3 Switch SVIs) | Router-on-a-stick: int g0/0.10encapsulation dot1q 10ip address 10.10.10.1 255.255.255.0 SVI (L3 switch): vlan 10int vlan 10ip address 10.10.10.1 255.255.255.0no shutip routing | show ip interface briefshow interfaces trunkshow ip route |
| ARP | (Normally no “enable” config; ARP is automatic on IPv4 Ethernet) | show ip arpclear ip arpping <ip> (forces ARP if needed) |
| EtherChannel (Static / PAgP / LACP) + L2 vs L3 | Member links (common pre-req): int range gi0/1-2switchport (for L2 EC) Static: channel-group 1 mode on PAgP: channel-group 1 mode desirable (or auto) LACP: channel-group 1 mode active (or passive) Port-channel (L2 trunk): int po1switchport mode trunkswitchport trunk allowed vlan 10,20 Port-channel (L3 routed): int range gi0/1-2no switchportchannel-group 1 mode activeint po1no switchportip address <ip> <mask> | show etherchannel summaryshow etherchannel port-channelshow interfaces port-channel 1show lacp neighbor / show pagp neighbor |
| RSTP (Rapid Spanning Tree) + roles/states | Enable rapid-PVST (Cisco): spanning-tree mode rapid-pvst Set root (per VLAN): spanning-tree vlan 10 root primaryspanning-tree vlan 10 root secondary (or priority <value>) Edge port hardening: int gi0/10spanning-tree portfastspanning-tree bpduguard enable | show spanning-tree summaryshow spanning-tree vlan 10show spanning-tree interface gi0/2 detaildebug spanning-tree events (use carefully) |
| FHRP / HSRP (default gateway redundancy) | HSRP on SVI/interface: int vlan 10standby 10 ip 10.10.10.1standby 10 priority 110standby 10 preempt Track uplink (optional): standby 10 track gi0/49 20 | show standby briefshow standbydebug standby (use carefully)show ip arpshow mac address-table dynamic |
| Static Routing (standard/default/floating/summary) (topic referenced in your notes set) | Next-hop: ip route 172.16.2.0 255.255.255.0 172.16.4.2 Exit-interface (ptp/serial): ip route 172.16.2.0 255.255.255.0 s0/0/0 Fully specified: ip route 172.16.2.0 255.255.255.0 s0/0/0 172.16.4.2 Default route: `ip route 0.0.0.0 0.0.0.0 <next-hop | exit-int><br>**Floating default**: ip route 0.0.0.0 0.0.0.0 5<br>**Summary**: ip route 200.23.16.0 255.255.240.0 ` |
| OSPFv2 (phases, packet types, DR/BDR) | Enable OSPF (network cmd): router ospf 1network 10.0.0.0 0.255.255.255 area 0 Enable per-interface: int gi0/0ip ospf 1 area 0 Set router-id: router ospf 1router-id 1.1.1.1 Passive interface: router ospf 1passive-interface gi0/1 Inject default route: router ospf 1default-information originate Reference bandwidth: router ospf 1auto-cost reference-bandwidth 10000 | show ip ospfshow ip ospf interface briefshow ip ospf neighborshow ip ospf databaseshow ip route ospfdebug ip ospf adj (use carefully) |